Looming test ver.2

Quick Guide (Review / Test Version)

Purpose

Windows application for presenting a visual looming stimulus to mice and logging trial timing to CSV. The stimulus design was based on the visual-looming shadow protocol described by Daviu et al. (2020).

Before starting

• Use Windows Extend display mode and place the stimulus monitor to the right of the PC display in Windows display settings.

• Open Bandicam and set the Record/Stop hotkey to F12.

• Choose the CSV destination using SELECT CSV FOLDER.

• Display the 10-cm calibration bar, measure it with a ruler, enter the measured value, and click APPLY & SAVE. Confirm that the final displayed length is 10.0 cm.

• Enter the Mouse ID before starting recording.

• Repeat physical-size calibration whenever the monitor or display configuration is changed.

Basic operation

| Key / button | Action |
| --- | --- |
| F9 / START RECORDING / TIME 0 | Start Bandicam recording and define the application's Time 0 |
| SPACE / TRIGGER LOOMING | Present one looming stimulus |
| Ctrl + R / NEW SESSION / RESET | Start a new session and reset trial count / Time 0 |
| ESC | Quit the application |
| STOP BANDICAM RECORDING | Send F12 to stop Bandicam recording |

Current stimulus settings

• Maximum: 5 trials per session. One trigger presents one looming stimulus.

• For the current 25.5-cm setup, disk diameter is set to approximately 1.7 cm -> 17.0 cm. This scales the 2 cm -> 20 cm stimulus described by Daviu et al. (2020) for a 30-cm monitor height by a distance ratio of 0.85.

• Sequence: small disk 3 s -> expansion over 2 s -> large disk 3 s -> gray screen.

• The background is gray (RGB 128, 128, 128), and the disk is centered on the stimulus display.

• Different monitor sizes/resolutions can be accommodated by the 10-cm calibration. If the monitor-to-test-floor distance changes, the stimulus-size setting should be reconsidered.

CSV output

The CSV records Mouse ID, Session ID, trial number, event, application video time, date/time, calibration scale, configured monitor-to-floor distance, and programmed disk diameters. Looming onset and end are logged for each trial.

Points I would like checked

• Does the stimulus appear on the intended monitor and remain centered?

• Does the physical-size calibration behave correctly?

• Are F9, SPACE, Ctrl+R, and ESC intuitive and safe during operation?

• Does Bandicam start/stop as expected, and is the timing sufficiently aligned for behavioral scoring?

• Is the CSV information sufficient for later analysis?

• Are there any confusing labels or steps in the user interface?

Notes

The application uses software hotkey synchronization with Bandicam; it is not hardware/frame-locked. Please treat this build as a review version and report any unexpected behavior before experimental use.

Reference

Daviu, N., Fuzesi, T., Rosenegger, D. G., Peringod, G., Simone, K., & Bains, J. S. (2020). Visual-looming Shadow Task with in-vivo Calcium Activity Monitoring to Assess Defensive Behaviors in Mice. Bio-protocol, 10(22), e3826. DOI: 10.21769/BioProtoc.3826.

The original protocol also provides the kpc-simone/shadow-graphics repository for Python code used to create the visual stimulus.